COMPREHENSIVE

DESIGN FOR MANUFACTURABILITY

GUIDELINES

CNC Machining Process

3-Axis  |  4-Axis  |  5-Axis Milling  |  CNC Turning  |  Secondary Operations

For Product Designers & Engineers

Document Version 1.0  |  March 2026

# Table of Contents

Table of Contents	2

1. Introduction and Purpose	4

1.1 How to Use This Document	4

1.2 Units and Conventions	4

2. General Design Principles	5

2.1 Fundamental DFM Rules	5

2.2 Cost Drivers in CNC Machining	5

3. Material Selection Guidelines	7

3.1 Metal Machinability Reference	7

3.2 Plastics Machinability Reference	8

4. Feature-Specific Design Guidelines	9

4.1 Holes	9

4.1.1 Through Holes	9

4.1.2 Blind Holes	9

4.1.3 Threaded Holes	10

4.2 Walls and Thin Features	11

4.3 Pockets and Cavities	11

4.3.1 Internal Corner Radius Selection Guide	12

4.4 Fillets and Radii	13

4.5 Chamfers	13

4.6 Threads (External and Internal)	15

4.6.1 Preferred Standard Thread Sizes	15

4.6.2 Thread Design Rules	15

4.7 Undercuts and Internal Features	15

4.8 Surface Finish	17

4.9 Slots and Grooves	17

5. Tolerancing Guidelines	19

5.1 General Tolerance Classes	19

5.2 Geometric Dimensioning & Tolerancing (GD&T)	19

6. 3-Axis Milling Guidelines	21

6.1 Design Constraints	21

6.2 Recommended Envelope and Capacity	21

7. 4-Axis Milling Guidelines	22

7.1 When to Design for 4-Axis	22

7.2 Design Rules for 4-Axis	22

8. 5-Axis Milling Guidelines	23

8.1 Advantages of 5-Axis Machining	23

8.2 Design Rules for 5-Axis	23

9. CNC Turning Guidelines	24

9.1 General Turning Design Rules	24

9.2 Turning Feature Design	24

9.3 Swiss-Type Turning	25

10. Workholding Considerations	26

10.1 Design-for-Fixturing Rules	26

11. Secondary Operations	27

11.1 Heat Treatment	27

11.2 Surface Treatments and Coatings	27

11.3 Grinding	28

11.4 EDM (Electrical Discharge Machining)	29

11.5 Deburring	29

12. Common DFM Violations Checklist	31

13. Cost Optimization Strategies	32

13.1 Design-Level Strategies	32

13.2 Material-Level Strategies	32

14. Drawing and Documentation Best Practices	33

14.1 Required Drawing Information	33

15. Reference Tables	34

15.1 Standard Metric Drill Sizes	34

15.2 Tap Drill Sizes (Metric Coarse Threads, ~75% Thread)	34

15.3 Standard End Mill Diameters	34

15.4 Unit Conversion Quick Reference	34

Appendix A: DFM Review Workflow	36

Appendix B: Glossary of CNC Machining Terms	37

# 1. Introduction and Purpose

This document provides comprehensive Design for Manufacturability (DFM) guidelines for CNC machining processes. It serves as a definitive reference for product designers and engineers to ensure that part designs are optimized for manufacturing efficiency, cost-effectiveness, and quality. These guidelines apply across the full spectrum of CNC operations including 3-axis, 4-axis, and 5-axis milling, CNC turning (standard and Swiss-type), as well as all common secondary operations such as heat treatment, surface finishing, and coatings.

Following these guidelines will help reduce manufacturing lead times, minimize tooling costs, improve part quality, reduce scrap rates, and ensure consistent repeatability across production runs.

## 1.1 How to Use This Document

Each section contains specific design rules organized by feature type or process. Recommended values represent standard best practice; achievable values represent what is possible with specialized tooling or setup, usually at higher cost. Designers should target recommended values wherever possible and consult with manufacturing engineering before specifying achievable limits.

## 1.2 Units and Conventions

Unless otherwise noted, all dimensions are in millimeters (mm). Tolerances are bilateral unless shown otherwise. Angles are in degrees. Surface roughness is expressed as Ra in micrometers. Where imperial equivalents are helpful, they are shown in parentheses.

# 2. General Design Principles

## 2.1 Fundamental DFM Rules

- Minimize the number of setups: Design parts so they can be machined in as few orientations as possible. Each additional setup adds cost, time, and potential tolerance stack-up.

- Design for standard tooling: Use standard drill sizes, standard thread sizes, and standard fillet radii that correspond to common tool diameters. Avoid features that require custom tooling.

- Avoid unnecessary tight tolerances: Specify only the tolerances that are functionally necessary. Tighter tolerances exponentially increase machining time and inspection costs.

- Ensure tool access: Every feature must be reachable by a cutting tool. Consider tool length, diameter, and approach angle when designing internal features.

- Minimize material removal: Design near-net-shape parts where possible. Excessive material removal wastes material, increases cycle time, and accelerates tool wear.

- Design for workholding: Ensure the part has adequate surfaces for clamping or fixturing. Include datum surfaces that can be reliably referenced across setups.

- Use uniform wall thicknesses: Uniform walls reduce the risk of distortion during machining and heat treatment. Gradual transitions are preferred over abrupt changes.

- Add fillets to internal corners: All internal corners require a fillet radius at minimum equal to the tool radius. Sharp 90-degree internal corners are physically impossible with rotary cutters.

## 2.2 Cost Drivers in CNC Machining

Understanding the primary cost drivers helps designers make informed trade-off decisions during the design process. The following table ranks common cost drivers from highest to lowest impact.

| Rank | Cost Driver | Impact | Design Mitigation |
| --- | --- | --- | --- |
| 1 | Number of setups | Very High | Design for single-setup machining; add fixture-friendly features |
| 2 | Tight tolerances (<0.025 mm) | Very High | Apply tight tolerances only to functional mating surfaces |
| 3 | Deep pockets / cavities | High | Limit depth-to-width ratio to 4:1 or less |
| 4 | Thin walls | High | Maintain minimum 0.8 mm (metals) or 1.5 mm (plastics) |
| 5 | Non-standard tooling | High | Use standard drill/end mill sizes and thread pitches |
| 6 | Complex surface finishes | Medium-High | Specify surface finish only where functionally required |
| 7 | Small internal radii | Medium | Use radii that match standard tool diameters (R1.5, R3, R6 mm) |
| 8 | Deep holes | Medium | Keep depth-to-diameter ratio below 10:1 where possible |
| 9 | Material hardness | Medium | Consider pre-hardened stock to avoid post-machining heat treatment |
| 10 | Part size / weight | Low-Medium | Design within standard machine envelopes |

# 3. Material Selection Guidelines

Material selection profoundly affects machinability, cost, tool life, achievable tolerances, and surface finish. The following table provides machinability guidance for common engineering materials.

## 3.1 Metal Machinability Reference

| Material | Machinability Rating | Typical Ra (um) | Notes |
| --- | --- | --- | --- |
| Aluminum 6061-T6 | Excellent | 0.4 - 1.6 | Preferred general-purpose alloy; excellent chip formation |
| Aluminum 7075-T6 | Excellent | 0.4 - 1.6 | Higher strength; slightly more tool wear than 6061 |
| Aluminum 2024-T3 | Good - Excellent | 0.8 - 1.6 | Gummy when annealed; machine in tempered condition |
| Brass C360 | Excellent | 0.4 - 0.8 | Free-machining; ideal for high-volume turned parts |
| Carbon Steel 1018 | Good | 0.8 - 3.2 | Low cost; good for general structural parts |
| Carbon Steel 1045 | Good | 1.6 - 3.2 | Medium carbon; hardenable; moderate tool wear |
| Alloy Steel 4140 | Fair - Good | 1.6 - 3.2 | Pre-hardened grades (28-32 HRC) available; common structural |
| Alloy Steel 4340 | Fair | 1.6 - 6.3 | High strength; requires carbide or ceramic tooling |
| Stainless Steel 303 | Good | 0.8 - 3.2 | Free-machining SS; preferred when corrosion resistance needed |
| Stainless Steel 304 | Fair | 1.6 - 3.2 | Work-hardens rapidly; use sharp tools, constant feed |
| Stainless Steel 316 | Fair | 1.6 - 6.3 | More difficult than 304; gummy chips; superior corrosion resistance |
| Stainless Steel 17-4 PH | Fair | 1.6 - 3.2 | Machine in Condition A; heat treat after; precipitation hardening |
| Titanium Ti-6Al-4V | Poor - Fair | 1.6 - 6.3 | Low thermal conductivity; requires rigid setup, flood coolant |
| Inconel 718 | Poor | 3.2 - 6.3 | Extreme tool wear; use ceramic inserts; low speeds |
| Copper C110 | Good | 0.4 - 1.6 | Soft; burrs easily; excellent thermal/electrical conductivity |
| Tool Steel D2 | Poor | 3.2 - 6.3 | Very hard (58-62 HRC); EDM often preferred for complex shapes |
| Tool Steel A2 | Fair | 1.6 - 3.2 | Air-hardening; machine in annealed state then heat treat |

## 3.2 Plastics Machinability Reference

| Material | Machinability | Typical Ra (um) | Notes |
| --- | --- | --- | --- |
| Delrin (POM / Acetal) | Excellent | 0.4 - 1.6 | Best overall plastic for CNC; excellent dimensional stability |
| Nylon 6/6 | Good | 0.8 - 3.2 | Absorbs moisture; may swell; dry before machining for best results |
| PEEK | Good | 0.8 - 1.6 | High cost; excellent for high-temp applications; carbide tools recommended |
| Polycarbonate (PC) | Good | 0.8 - 1.6 | Transparent; stress-cracks with some coolants; use air blast |
| Acrylic (PMMA) | Good | 0.2 - 0.8 | Brittle; cracks if clamped too tightly; excellent optical clarity |
| UHMW-PE | Fair | 1.6 - 3.2 | Very flexible; deflects under tool pressure; use sharp tools |
| PTFE (Teflon) | Fair | 1.6 - 6.3 | Deforms under clamping; cold creep; vacuum fixturing often needed |
| ABS | Good | 0.8 - 3.2 | Easy to machine; melts at high speeds; use moderate RPM |
| PEI (Ultem) | Good | 0.8 - 1.6 | Amorphous; good dimensional stability; aerospace applications |
| Garolite G-10/FR4 | Fair | 3.2 - 6.3 | Abrasive fiberglass; rapid tool wear; diamond-coated tools preferred |

Material Selection Tip When in doubt, Aluminum 6061-T6 offers the best combination of machinability, strength, corrosion resistance, and cost for prototyping and general-purpose applications. For plastic prototypes, Delrin (Acetal/POM) is the gold standard for CNC machining due to its excellent dimensional stability and chip formation.

# 4. Feature-Specific Design Guidelines

This section provides detailed design rules for each common CNC-machined feature type. Each subsection includes recommended values, achievable limits, and the rationale behind each guideline.

## 4.1 Holes

Holes are among the most common features in CNC machined parts. Proper hole design significantly affects cycle time, tool life, and achievable quality.

### 4.1.1 Through Holes

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Minimum diameter | 1.0 mm (0.040") | 0.5 mm (0.020") | Smaller diameters require specialized micro-drills |
| Maximum depth-to-diameter ratio | 6:1 | 10:1 | Beyond 6:1 requires peck drilling; beyond 10:1 consider gun drilling |
| Preferred diameters | Standard drill sizes | Any | Use standard fractional, letter, or metric drill sizes to avoid reaming |
| Diameter tolerance (standard) | +/- 0.05 mm | +/- 0.013 mm | Tighter tolerance requires reaming or boring |
| Diameter tolerance (reamed) | +/- 0.013 mm | +/- 0.005 mm | H7 tolerance class achievable with reaming |
| Position tolerance | +/- 0.05 mm | +/- 0.013 mm | Tighter requires precision boring or jig boring |
| Edge distance (min) | Diameter x 1.5 | Diameter x 1.0 | Less than 1x diameter risks breakout and deformation |
| Hole-to-hole spacing (min) | Diameter x 2.0 | Diameter x 1.5 | Closer spacing risks thin wall deflection between holes |
| Entry/exit surface | Perpendicular | Up to 5 deg | Angled surfaces cause drill walk; use spot drill or flat |

### 4.1.2 Blind Holes

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Bottom geometry | 118 deg cone (drill point) | Flat bottom | Flat bottom requires end mill; adds cost and time |
| Depth tolerance | +/- 0.13 mm | +/- 0.05 mm | Measured to full diameter, not drill point |
| Max depth-to-diameter | 4:1 | 8:1 | Chip evacuation is harder in blind holes |
| Minimum corner radius at bottom | 0.5 x diameter | 0.1 mm | Flat bottom requires separate end mill operation |
| Minimum remaining wall thickness | 0.5 mm | 0.3 mm | Thin floors prone to deflection and breakthrough |

Design Tip – Holes Wherever possible, design through holes instead of blind holes. Through holes are faster to machine, easier to deburr, easier to inspect, and produce better surface finishes. If a blind hole is functionally required, accept the standard 118-degree drill-point bottom unless a flat bottom is absolutely necessary.

### 4.1.3 Threaded Holes

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Minimum thread size | M3 / #4-40 | M1.6 / #0-80 | Smaller threads are fragile and limit torque capacity |
| Thread engagement depth | 1.5 x diameter | 3.0 x diameter | Beyond 2x adds minimal strength; increases tap breakage risk |
| Thread relief at bottom | 1-2 pitches clearance | 0.5 pitch | Relief prevents tap bottoming and breakage |
| Preferred thread form | Standard metric or UNC/UNF | Custom | Standard threads use stock taps; custom requires thread milling |
| Thread class (metric) | 6H | 4H | Tighter classes require thread milling or precision tapping |
| Thread class (imperial) | 2B | 3B | 3B requires controlled-lead tapping or thread milling |
| Minimum wall to thread | 2.0 x pitch | 1.0 x pitch | Thin walls deform under tapping forces |
| Chamfer at thread entry | 0.5 mm x 45 deg | 0.25 mm x 45 deg | Chamfer guides the fastener and protects thread start |

## 4.2 Walls and Thin Features

Thin walls are one of the most common causes of machining problems. Inadequate wall thickness leads to vibration (chatter), deflection, distortion, and in extreme cases, part failure during machining.

| Material Category | Min Wall (Recommended) | Min Wall (Achievable) | Max Unsupported Height-to-Thickness Ratio |
| --- | --- | --- | --- |
| Aluminum alloys | 0.8 mm (0.031") | 0.5 mm (0.020") | 15:1 |
| Carbon & alloy steels | 0.8 mm (0.031") | 0.5 mm (0.020") | 12:1 |
| Stainless steels | 1.0 mm (0.040") | 0.5 mm (0.020") | 10:1 |
| Titanium alloys | 1.0 mm (0.040") | 0.6 mm (0.024") | 8:1 |
| Copper alloys (soft) | 1.0 mm (0.040") | 0.5 mm (0.020") | 10:1 |
| Engineering plastics | 1.5 mm (0.060") | 0.8 mm (0.031") | 20:1 |
| Soft plastics (PE, PTFE) | 2.0 mm (0.079") | 1.0 mm (0.040") | 12:1 |

Critical Warning – Thin Walls Thin walls adjacent to deep pockets are the single most common DFM violation. The combination of thin wall + deep pocket creates a lever arm that amplifies vibration. If thin walls are unavoidable, use light cuts with high spindle speeds, add temporary support ribs that are removed in a final pass, or consider stress-relief heat treatment between roughing and finishing operations.

## 4.3 Pockets and Cavities

Pockets are enclosed material-removal features. Their design directly impacts tool selection, cycle time, and achievable quality.

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Min internal corner radius | 1/3 of pocket depth | Tool radius + 0.1 mm | Must match or exceed end mill radius; include 0.1 mm clearance |
| Max depth-to-width ratio | 4:1 | 6:1 | Deep narrow pockets require long-reach tools that deflect |
| Min pocket width | 2.0 x tool diameter | 1.0 x tool diameter | Narrow pockets limit chip evacuation and tool engagement |
| Floor flatness | 0.05 mm over 50 mm | 0.013 mm over 50 mm | Depends on tool deflection and machine rigidity |
| Floor-to-wall transition | Fillet = tool radius | 0.5 mm min | Sharp corners impossible; design fillet to match standard tool |
| Pocket bottom thickness | Min 1.0 mm (metal) | 0.5 mm (metal) | Thin bottoms deflect and risk breakthrough |
| Draft angle on walls | 0 deg (vertical) | 0 deg (vertical) | CNC can cut vertical walls; draft not needed unlike injection molding |
| Step-down transitions | Fillet R 0.5 mm min | R 0.25 mm | Smooth transitions reduce stress concentration and tool loading |

### 4.3.1 Internal Corner Radius Selection Guide

The internal corner radius is one of the most critical dimensions in pocket design. It determines the minimum tool diameter, which in turn affects achievable depth and surface finish. Use the following guide to select appropriate corner radii.

| Pocket Depth | Minimum Corner Radius | Standard Tool Diameter | Tool L/D Ratio |
| --- | --- | --- | --- |
| Up to 6 mm | R 1.5 mm | 3 mm end mill | 2:1 |
| Up to 12 mm | R 3.0 mm | 6 mm end mill | 2:1 |
| Up to 25 mm | R 5.0 mm | 10 mm end mill | 2.5:1 |
| Up to 38 mm | R 6.0 mm | 12 mm end mill | 3.2:1 |
| Up to 50 mm | R 8.0 mm | 16 mm end mill | 3.1:1 |
| Up to 75 mm | R 10.0 mm | 20 mm end mill | 3.75:1 |
| Up to 100 mm | R 12.0 mm | 25 mm end mill | 4:1 |

Internal Corner Radius Tip Always specify the internal corner radius as slightly larger than the tool radius (at least 0.1 mm or 10% larger). This prevents the tool from engaging the full 180 degrees of its circumference in corners, which causes tool deflection, chatter, and premature wear. For example, if using a 6 mm end mill (R3), specify R3.5 mm on the drawing.

## 4.4 Fillets and Radii

Fillets and radii are critical for both manufacturability and structural integrity. Internal fillets are limited by tool geometry, while external radii offer more design freedom.

| Feature Type | Recommended Radius | Minimum Achievable | Notes |
| --- | --- | --- | --- |
| Internal vertical fillets | R 3.0 mm | R 0.25 mm | Must match cutter radius; controls minimum tool size |
| Internal floor fillets | R 1.0 mm | R 0.25 mm | Ball end mill radius; affects pocket-floor transition |
| External edges | R 0.5 mm (chamfer preferred) | Sharp (burr removed) | Sharp external edges are achievable but fragile |
| Fillet between ribs and walls | R 1.0 mm | R 0.5 mm | Reduces stress concentration at junctions |
| Undercut fillets | R 1.0 mm | R 0.5 mm | Requires T-slot cutter or special form tool |
| Blend radius between surfaces | R 3.0 mm or greater | R 0.5 mm | Larger radii allow faster feed rates and better finish |

Consistency Tip Use a maximum of 2-3 different fillet radii throughout a part. This minimizes tool changes and keeps cycle times low. Pick standard radii that correspond to common tool diameters: R1.5, R3.0, R5.0, R6.0, R8.0, R10.0, and R12.0 mm.

## 4.5 Chamfers

Chamfers are preferred over fillets for external edges because they can be cut quickly with a standard chamfer mill in a single pass. They also ease assembly, remove burrs, and reduce stress concentrations on external edges.

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Standard chamfer angle | 45 degrees | Any angle | 45 deg is default; other angles require angle-specific tools |
| Minimum chamfer size | 0.25 mm x 45 deg | 0.1 mm x 45 deg | Very small chamfers hard to inspect and maintain consistency |
| Maximum chamfer size | 3.0 mm x 45 deg | Limited by geometry | Large chamfers may require multiple passes |
| Chamfer on hole entry | 0.5 mm x 45 deg | 0.25 mm x 45 deg | Aids fastener insertion and deburrs hole edge |
| Chamfer tolerance | +/- 0.25 mm | +/- 0.1 mm | Chamfer size is difficult to measure precisely |

## 4.6 Threads (External and Internal)

Thread design affects both the machining process and the functionality of the final assembly. Use standard thread sizes whenever possible to leverage off-the-shelf taps, dies, and thread mills.

### 4.6.1 Preferred Standard Thread Sizes

| Metric (Coarse) | Metric (Fine) | Imperial (UNC) | Imperial (UNF) |
| --- | --- | --- | --- |
| M3 x 0.5 | M3 x 0.35 | #4-40 | #4-48 |
| M4 x 0.7 | M4 x 0.5 | #6-32 | #6-40 |
| M5 x 0.8 | M5 x 0.5 | #8-32 | #8-36 |
| M6 x 1.0 | M6 x 0.75 | #10-24 | #10-32 |
| M8 x 1.25 | M8 x 1.0 | 1/4-20 | 1/4-28 |
| M10 x 1.5 | M10 x 1.25 | 5/16-18 | 5/16-24 |
| M12 x 1.75 | M12 x 1.25 | 3/8-16 | 3/8-24 |
| M16 x 2.0 | M16 x 1.5 | 1/2-13 | 1/2-20 |
| M20 x 2.5 | M20 x 1.5 | 5/8-11 | 5/8-18 |
| M24 x 3.0 | M24 x 2.0 | 3/4-10 | 3/4-16 |

### 4.6.2 Thread Design Rules

- Use coarse threads as default: Coarse threads are stronger, more forgiving of tolerance variation, and less prone to cross-threading. Use fine threads only when vibration resistance or fine adjustment is needed.

- Thread engagement depth: 1.5x to 2.0x the nominal thread diameter provides full strength in steel and aluminum. Beyond 2.0x adds negligible strength but increases tap breakage risk.

- Thread relief: Allow at least 1.5 to 2 pitches of unthreaded depth at the bottom of blind tapped holes for chip clearance and to prevent tap bottoming.

- Thread milling vs tapping: Thread milling is preferred for large threads (> M12), hard materials (> 35 HRC), blind holes with limited depth, and when thread quality is critical. Tapping is faster and preferred for smaller threads in softer materials.

- Minimum wall around threads: Maintain at least 2x the thread pitch between the thread major diameter and any adjacent wall or feature.

## 4.7 Undercuts and Internal Features

Undercuts are features that cannot be reached by a standard end mill approaching from directly above. They require specialized tooling (T-slot cutters, lollipop cutters, or keyway cutters) and add significant cost and complexity.

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Avoid undercuts if possible | Yes | N/A | Redesign to eliminate undercuts wherever possible |
| Standard O-ring groove width | Per O-ring spec | Per spec | Use standard AS568 groove dimensions |
| T-slot width | Standard T-slot sizes | Custom | Standard T-slots match standard T-slot cutters |
| Undercut depth | < 2x tool neck diameter | 3x tool neck | Deep undercuts require long slender tools that deflect |
| Min access opening | Tool diameter + 2 mm | Tool diameter + 0.5 mm | Tool must pass through opening to reach undercut |
| Dovetail angle | 45 or 60 degrees | Any standard angle | Match standard dovetail cutter angles |

DFM Recommendation If an undercut is required for an O-ring groove, retaining ring groove, or similar functional feature, specify it with standard dimensions that match commercially available cutters. Custom undercut profiles are expensive and have long lead times for tooling. Wherever possible, redesign the part so undercut features become accessible from a standard tool direction.

## 4.8 Surface Finish

Surface finish is specified as Ra (arithmetic average roughness) in micrometers. The achievable finish depends on the material, tool condition, machining parameters, and feature geometry. Specifying a finer finish than necessary significantly increases cost.

| Ra Value (um) | Ra Value (uin) | Typical Process | Application | Relative Cost |
| --- | --- | --- | --- | --- |
| 6.3 | 250 | Rough milling / turning | Non-critical surfaces, hidden faces | 1x (baseline) |
| 3.2 | 125 | Standard milling / turning | General machined surfaces (as-machined default) | 1x |
| 1.6 | 63 | Finish milling / turning | Mating surfaces, bearing surfaces, visible surfaces | 1.5x |
| 0.8 | 32 | Fine finish pass / grinding | Precision fits, seal surfaces, hydraulic bores | 2-3x |
| 0.4 | 16 | Grinding / lapping / polishing | Precision bearing journals, optical mounting surfaces | 4-6x |
| 0.2 | 8 | Lapping / superfinishing | Gauge blocks, precision optics mounts | 8-15x |
| 0.1 | 4 | Superfinishing / polishing | Mirror finish, semiconductor equipment | 15-30x |

Surface Finish Guidance The industry-standard as-machined finish is Ra 3.2 um (125 uin). Unless otherwise specified, this is what most CNC shops will deliver. Only callout specific surface finishes on surfaces where they are functionally required (sealing surfaces, bearing surfaces, mating interfaces). Leaving non-critical surfaces at as-machined finish significantly reduces cost.

## 4.9 Slots and Grooves

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Minimum slot width | 1.5 mm | 0.5 mm | Narrow slots require small fragile tools |
| Max depth-to-width ratio | 4:1 | 8:1 | Deep narrow slots limit chip evacuation |
| Slot end radius | = Tool radius | Specified | Slotting with end mill leaves rounded ends; square ends need EDM |
| Minimum web between slots | = Slot width | 0.5 x slot width | Thin webs deflect and chatter |
| Keyway width | Standard keyway sizes | Custom | Standard widths match standard broaches/end mills |
| Groove width (external) | 1.5 mm min | 0.5 mm | On turned parts; narrow grooves require form tools |

# 5. Tolerancing Guidelines

Tolerances have an outsized impact on manufacturing cost. The following guidelines help designers specify tolerances that achieve the required function without unnecessary precision.

## 5.1 General Tolerance Classes

| Tolerance Class | Linear Tolerance | Angular Tolerance | Typical Application | Relative Cost |
| --- | --- | --- | --- | --- |
| Coarse | +/- 0.5 mm | +/- 1.0 deg | Non-functional, clearance features | 0.5x |
| Medium (Standard) | +/- 0.13 mm (+/- 0.005") | +/- 0.5 deg | General machined features (default) | 1x |
| Precise | +/- 0.05 mm (+/- 0.002") | +/- 0.25 deg | Mating surfaces, bearing fits | 2x |
| High Precision | +/- 0.025 mm (+/- 0.001") | +/- 0.10 deg | Precision assemblies, optical mounts | 3-5x |
| Ultra Precision | +/- 0.013 mm (+/- 0.0005") | +/- 0.05 deg | Gauge-quality, metrology equipment | 5-10x |
| Grinding / Lapping | +/- 0.005 mm (+/- 0.0002") | N/A | Seal surfaces, gauge blocks | 10-25x |

## 5.2 Geometric Dimensioning & Tolerancing (GD&T)

GD&T provides a more precise way to communicate design intent than simple linear tolerances. The following table summarizes achievable values for common GD&T callouts on CNC machined parts.

| GD&T Symbol | Characteristic | CNC Typical | CNC Best | Notes |
| --- | --- | --- | --- | --- |
| ◎ | Flatness | 0.05 mm / 100 mm | 0.013 mm / 100 mm | Affected by part thickness and clamping stress |
| ○ | Circularity (Roundness) | 0.013 mm | 0.005 mm | Turned features; depends on spindle runout |
| // | Parallelism | 0.025 mm / 100 mm | 0.010 mm / 100 mm | Between two machined faces |
| ⊥ | Perpendicularity | 0.025 mm / 100 mm | 0.010 mm / 100 mm | Between features in same setup is tightest |
| ⬡ | Cylindricity | 0.025 mm | 0.008 mm | Requires precision turning or grinding |
| ⌖ | Position (Hole) | 0.05 mm | 0.013 mm | True position at MMC; depends on fixturing |
| ⇑ | Concentricity | 0.025 mm | 0.008 mm | Co-axial features; best in single setup |
| ∥ | Symmetry | 0.05 mm | 0.013 mm | Symmetric features about datum plane |
| ∿ | Profile of a Surface | 0.05 mm | 0.013 mm | 3D surface tolerance; 5-axis capability helps |
| ↗ | Total Runout | 0.025 mm | 0.008 mm | Composite of circularity + concentricity |

Tolerance Strategy Apply the 80/20 rule: 80% of features should use the general (medium) tolerance, 15% may need precise tolerances, and only 5% or fewer should require high precision. Mark tight tolerances clearly on drawings and include a note explaining the functional reason. This communicates intent to the machinist and helps the shop optimize their approach.

# 6. 3-Axis Milling Guidelines

3-axis milling is the most common and cost-effective CNC milling process. The tool moves in X, Y, and Z while the workpiece remains fixed. All features must be accessible from a single direction per setup.

## 6.1 Design Constraints

- Line-of-sight access: Every feature must be visible and accessible from directly above (along the tool axis). No overhanging or undercut features are possible without additional setups.

- Maximum of 6 accessible faces: A prismatic part can be machined on all six faces, but each face requires a separate setup (flip). Minimize the number of faces requiring machining.

- 2.5D geometry preferred: Features with constant cross-section in Z (pockets, holes, slots) are fastest to machine. True 3D sculptured surfaces require ball-end milling with fine stepovers, significantly increasing cycle time.

- Vertical walls are standard: Walls perpendicular to the machine table are natural for 3-axis milling. Angled walls require ball-end mills or tapered end mills.

- Floor fillets depend on tool: The transition between a vertical wall and a horizontal floor always has a fillet equal to the ball nose radius of the tool used.

## 6.2 Recommended Envelope and Capacity

| Parameter | Typical Range | Notes |
| --- | --- | --- |
| Maximum part size | 1000 x 500 x 500 mm | Standard VMC; larger parts require horizontal or gantry mills |
| Minimum feature size | 0.5 mm | Limited by smallest available end mill |
| Typical positional accuracy | +/- 0.025 mm | Modern VMC with probing |
| Typical surface finish (milling) | Ra 1.6 - 3.2 um | As-machined with new tooling |
| Maximum tool length (practical) | 150 mm (6") | Longer tools deflect; affects accuracy and finish |
| Typical number of setups | 2-6 | Top + bottom minimum; more for features on multiple faces |

# 7. 4-Axis Milling Guidelines

4-axis milling adds a rotary axis (typically the A-axis, rotating around X) to the three linear axes. This enables machining features on multiple faces of a part in a single setup, particularly for cylindrical or prismatic parts mounted on a rotary table or indexer.

## 7.1 When to Design for 4-Axis

- Parts with features on 3 or more faces that would otherwise require multiple setups.

- Cylindrical parts with features at multiple angular positions (ports, flats, holes).

- Parts requiring continuous rotary machining (helical features, cam profiles, turbine blades).

- Parts where inter-feature angular position tolerance is critical (easier to hold in one setup).

## 7.2 Design Rules for 4-Axis

| Parameter | Guideline | Notes |
| --- | --- | --- |
| Rotary axis clearance | Ensure tool clears rotary table at all angles | Check for interference between tool, part, and fixture |
| Angular position tolerance | +/- 0.05 deg typical | Indexing accuracy of rotary table |
| Feature orientation | Align features to cardinal angles (0, 90, 180, 270 deg) | Indexed positions are most accurate; continuous is less precise |
| Part balance | Balanced mass distribution preferred | Unbalanced parts cause vibration at high rotary speeds |
| Wrap-around features | Continuous 4-axis capable | Helical grooves, cam profiles possible |
| Fixturing | Central bore or shaft for mandrel mounting | Design a datum bore or shaft for rotary mounting |

# 8. 5-Axis Milling Guidelines

5-axis milling adds two rotary axes to the three linear axes, enabling the tool to approach the workpiece from virtually any direction. This dramatically expands geometric freedom and often allows complex parts to be machined in a single setup.

## 8.1 Advantages of 5-Axis Machining

- Single-setup machining of complex parts with features on multiple faces and angles.

- Shorter tools due to optimal tool orientation, resulting in better rigidity, accuracy, and surface finish.

- Ability to machine undercuts, compound angles, and sculptured surfaces impossible with 3-axis.

- Improved surface finish on 3D contours through constant tool-surface contact angle.

- Reduced number of fixtures and setups, improving positional accuracy between features.

## 8.2 Design Rules for 5-Axis

| Parameter | Guideline | Notes |
| --- | --- | --- |
| Tool approach angle | Min 15 deg from surface tangent | Shallow angles cause poor cutting and surface finish |
| Concave surface min radius | R > tool radius + 10% | Tool must fit within concavity with clearance |
| Impeller blade thickness | > 0.8 mm for aluminum | Thin blades deflect under tool load |
| Compound angle holes | Fully supported | 5-axis tilts tool to drill perpendicular to angled surfaces |
| Turbine blade twist | Continuous 5-axis required | Ruled surfaces possible with 3+2; sculpted requires full 5-axis |
| Multi-face datum alignment | +/- 0.013 mm achievable | All features referenced to single datum in one setup |
| Collision avoidance | Min 5 mm tool-fixture clearance | CAM simulation essential; design fixturing with tool paths in mind |
| 3+2 vs. simultaneous 5-axis | 3+2 for indexed features | 3+2 (positional) is more rigid and accurate than continuous 5-axis |

5-Axis Cost Consideration 5-axis machining has a significantly higher hourly rate than 3-axis (typically 1.5x to 2.5x). However, it often reduces total cost by eliminating multiple setups, fixtures, and the tolerance stack-up between setups. Consider 5-axis when a part would require 4+ setups on a 3-axis machine, or when inter-feature positional accuracy is critical.

# 9. CNC Turning Guidelines

CNC turning produces rotationally symmetric parts using a rotating workpiece and a stationary or live cutting tool. Parts are held in a chuck or collet and features are generated along and around the axis of rotation.

## 9.1 General Turning Design Rules

| Parameter | Recommended | Achievable | Notes |
| --- | --- | --- | --- |
| Min turned diameter | 3.0 mm | 0.5 mm (Swiss lathe) | Small diameters require Swiss-type machines or collet chucks |
| Max length-to-diameter ratio (unsupported) | 4:1 | 8:1 | Beyond 4:1 requires tailstock support or steady rest |
| Max L/D with tailstock | 10:1 | 20:1 | Tailstock center supports the free end |
| Diameter tolerance | +/- 0.025 mm | +/- 0.005 mm | Tight tolerances require precision boring or grinding |
| Length tolerance | +/- 0.05 mm | +/- 0.013 mm | Facing operations; limited by tool nose radius compensation |
| Concentricity (between diameters) | 0.025 mm TIR | 0.005 mm TIR | Best when machined in single chucking |
| Surface finish (OD turning) | Ra 1.6 um | Ra 0.4 um | Fine finish requires sharp inserts, low feed, high speed |
| Surface finish (boring) | Ra 1.6 um | Ra 0.4 um | Boring bar deflection limits achievable finish at depth |
| Minimum bore diameter | 6 mm | 3 mm | Limited by boring bar rigidity at depth |
| Max bore depth-to-diameter | 4:1 | 8:1 | Deep bores require dampened boring bars |

## 9.2 Turning Feature Design

- Chamfer all external sharp edges: 0.5 mm x 45 deg minimum. Chamfers facilitate assembly and eliminate hazardous sharp edges.

- Include relief grooves before shoulders: A relief groove (undercut) at shoulders allows turning tools to reach full depth and ensures clean right-angle transitions.

- Use standard insert nose radii: Common values are R 0.4, R 0.8, R 1.2, and R 1.6 mm. The tool nose radius appears as a fillet at corners and shoulders.

- Center drill both ends of shafts: Center holes allow support between centers for grinding and inspection. Specify 60-degree center drill size A or B.

- Knurling: Standard diamond and straight knurl patterns are available. Specify knurl pitch (e.g., 0.8 mm) and pattern. Knurled diameter grows by approximately 0.5x the pitch on radius.

- Design for collet chucking: Parts requiring high concentricity should include a precision reference diameter for collet gripping.

## 9.3 Swiss-Type Turning

Swiss-type (sliding headstock) lathes are specialized for small-diameter, long, slender parts. The workpiece is supported by a guide bushing directly adjacent to the cutting zone, enabling extremely tight tolerances on long thin parts.

| Parameter | Typical Capability | Notes |
| --- | --- | --- |
| Part diameter range | 1 - 32 mm | Most common 12 - 20 mm capacity |
| Length-to-diameter ratio | Up to 30:1 | Guide bushing supports near the cut zone |
| Diameter tolerance | +/- 0.005 mm | Precision collet and guide bushing |
| Surface finish | Ra 0.4 um | Excellent due to guide bushing rigidity |
| Live tooling | Cross drilling, milling, polygon turning | Eliminates secondary operations |
| Ideal for | Fasteners, pins, shafts, medical, connectors | High-volume, small-diameter precision parts |

# 10. Workholding Considerations

Proper workholding is essential for achieving dimensional accuracy and surface quality. Part design should always account for how the part will be fixtured during each machining operation.

## 10.1 Design-for-Fixturing Rules

- Flat datum surfaces: Include at least one large flat surface for the first setup. This surface contacts the vise jaw or fixture plate and serves as the primary datum.

- Parallel opposing faces: Two parallel faces enable vise clamping. Ensure adequate area and parallelism for secure gripping.

- Minimum clamping area: Provide at least 10 mm of clamping surface in the vise direction. Parts too thin or too small require custom fixtures.

- Avoid interrupted clamping surfaces: Holes, pockets, or slots on clamping faces reduce grip force and cause uneven pressure.

- Soft jaw features: For round or irregular parts, include a datum diameter or flat that soft jaws can be machined to match.

- Sacrificial tabs/bridges: For thin or fragile parts, design sacrificial material that holds the part during machining and is removed afterward.

- Vacuum fixturing features: For thin plates, ensure one face is flat and smooth for vacuum chuck sealing. Seal path must be uninterrupted.

Workholding Tip If you are unsure how a part will be held, add a note on the drawing asking the machine shop for a fixturing plan review. Many DFM issues can be resolved by adding small non-functional features (locating holes, alignment flats, or clamping tabs) that dramatically simplify workholding.

# 11. Secondary Operations

Secondary operations are processes performed after primary CNC machining to achieve properties or features not possible with machining alone. Proper design consideration for secondary operations prevents costly rework and dimensional issues.

## 11.1 Heat Treatment

| Process | Typical Materials | Effect | DFM Considerations |
| --- | --- | --- | --- |
| Through Hardening | 4140, 4340, 1045 | Increases hardness and strength throughout | Machine before HT; leave 0.25-0.5 mm stock for finish grinding after HT due to distortion |
| Case Hardening (Carburize) | 8620, 1018, 9310 | Hard surface, tough core | Design masking areas for surfaces that must remain soft; min 2 mm wall for carburizing |
| Nitriding | 4140, 4340, Nitralloy | Hard surface, minimal distortion | Excellent dimensional stability; minimal stock allowance needed |
| Induction Hardening | 1045, 4140, 4340 | Localized surface hardening | Design features to accommodate induction coil geometry; uniform cross-section preferred |
| Stress Relief | All steels, aluminum | Removes residual machining stress | Critical for tight-tolerance parts; perform between roughing and finishing |
| Aging / Precipitation Hardening | 17-4 PH, 7075-T6, A286 | Increases strength via precipitate formation | Machine in solution-treated condition; age after for optimal properties |
| Annealing | All metals | Softens material, relieves stress | Used before machining hard stock or between heavy machining operations |
| Cryogenic Treatment | Tool steels, bearing steels | Converts retained austenite; improves wear | Dimensional changes minimal; improves long-term stability |

Heat Treatment Warning Heat treatment causes dimensional changes. Case hardening and through hardening typically cause 0.05-0.25 mm of growth or distortion. For precision features, leave grinding stock (0.25-0.5 mm per side) and finish to final dimensions after heat treatment. Always specify the desired final hardness and depth on the drawing.

## 11.2 Surface Treatments and Coatings

| Process | Typical Materials | Thickness Added | DFM Considerations |
| --- | --- | --- | --- |
| Anodize Type II (Standard) | Aluminum alloys | 12-25 um total (50% penetrates, 50% builds up) | Grows ~0.013 mm per side; mask threads and tight-tolerance bores |
| Anodize Type III (Hard) | Aluminum alloys | 25-75 um total | Grows ~0.025-0.038 mm per side; account in tolerances; very hard (60-70 HRC equivalent) |
| Chromate Conversion (Alodine) | Aluminum alloys | < 1 um | No dimensional change; excellent for electrical conductivity + corrosion protection |
| Electroless Nickel | Steel, aluminum, copper | 5-75 um | Very uniform coating; builds up 0.005-0.075 mm per side; excellent corrosion resistance |
| Hard Chrome Plating | Steel, stainless | 25-250 um | Builds up 0.025-0.25 mm per side; excellent wear resistance; must account in dimensions |
| Zinc Plating | Steel | 5-25 um | Builds up 0.005-0.025 mm; hydrogen embrittlement concern on high-strength steels |
| Black Oxide | Steel, stainless | < 2 um | Negligible dimensional change; provides mild corrosion resistance and appearance |
| Passivation | Stainless steel | None (removes iron) | No dimensional change; improves natural corrosion resistance of stainless steel |
| Powder Coat | Steel, aluminum | 50-150 um | Significant build-up; mask all precision surfaces and threads |
| Nickel Plating (electrolytic) | Steel, copper | 10-50 um | Moderate build-up; less uniform than electroless; lower cost |
| Tin Plating | Copper, steel | 5-25 um | Solderability; used for electrical contacts and connectors |
| PTFE Impregnated Anodize | Aluminum | 12-50 um | Low friction coating; ideal for sliding surfaces; accounts same as Type II/III |

## 11.3 Grinding

Grinding is used to achieve tolerances and surface finishes beyond standard CNC machining capability, typically after heat treatment.

| Grinding Type | Tolerance | Surface Finish | DFM Considerations |
| --- | --- | --- | --- |
| Surface (flat) grinding | +/- 0.005 mm | Ra 0.2 - 0.8 um | Leave 0.25 mm grinding stock; ensure flatness datum; min 3 mm part thickness |
| Cylindrical OD grinding | +/- 0.005 mm | Ra 0.2 - 0.4 um | Include center holes for between-centers grinding; leave 0.2 mm stock on diameter |
| Cylindrical ID grinding | +/- 0.008 mm | Ra 0.4 - 0.8 um | Min bore diameter 6 mm; max depth 4x diameter; leave 0.15-0.25 mm stock |
| Centerless grinding | +/- 0.005 mm | Ra 0.2 - 0.4 um | Excellent for high-volume cylindrical parts; needs uninterrupted OD |
| Jig grinding | +/- 0.003 mm | Ra 0.2 - 0.4 um | Precision hole location and profile; very high cost |

## 11.4 EDM (Electrical Discharge Machining)

EDM is used for features that are impossible or impractical with conventional CNC machining, particularly in hardened steels and complex internal geometries.

| EDM Type | Tolerance | Surface Finish | Best Applications |
| --- | --- | --- | --- |
| Wire EDM | +/- 0.005 mm | Ra 0.4 - 1.6 um | Through profiles, gears, splines, sharp internal corners, hardened steel |
| Sinker (Ram) EDM | +/- 0.013 mm | Ra 0.8 - 3.2 um | Blind cavities, ribs, textures, complex 3D shapes in hard materials |
| Hole drilling EDM | +/- 0.025 mm | Ra 3.2 - 6.3 um | Small deep holes, cooling holes in turbine blades, start holes for wire EDM |

EDM Design Tip Wire EDM can produce perfectly sharp internal corners, which is impossible with rotary cutters. If sharp internal corners are functionally required (e.g., for splines, keyways, or gear teeth), specify wire EDM for those features. However, note that wire EDM requires through-features (the wire must enter and exit the workpiece). For blind sharp-cornered pockets, sinker EDM is required.

## 11.5 Deburring

All machined edges produce burrs to some extent. Design choices can significantly affect deburring cost and ease.

- Intersecting holes: The intersection of two holes creates a burr on the inside of the smaller hole that is extremely difficult to remove. Avoid or minimize intersecting holes; if required, specify acceptable burr height.

- Cross-drilled holes: Port intersections on hydraulic manifolds are a common burr challenge. Design for thermal deburring (TEM) or abrasive flow machining (AFM) access if critical.

- Chamfer all accessible edges: A 0.25-0.5 mm chamfer on the drawing preemptively removes the edge that would otherwise carry a burr.

- Tumble-friendly geometry: If parts will be tumble-deburred, avoid features that trap media (deep blind holes, narrow slots, small pockets).

- Specify burr height tolerance: For critical applications (medical, aerospace), specify maximum allowable burr height (e.g., 0.05 mm max).

# 12. Common DFM Violations Checklist

Use this checklist to review designs before release to manufacturing. Each item represents a frequent design error that increases cost, delays delivery, or reduces quality.

| # | Violation | Impact | Resolution |
| --- | --- | --- | --- |
| 1 | Sharp internal corners (zero radius) | Impossible to machine with rotary cutters | Add fillet radius >= tool radius + 10% clearance |
| 2 | Deep pockets with small radii | Requires long, thin tools that deflect and chatter | Increase radius or reduce depth; depth:width <= 4:1 |
| 3 | Thin walls adjacent to deep pockets | Vibration, deflection, potential wall failure | Increase wall thickness or reduce pocket depth |
| 4 | Unnecessarily tight tolerances on non-functional features | Dramatically increases cycle time and cost | Apply tight tolerances only where functionally required |
| 5 | Non-standard hole sizes | Requires reaming or interpolation instead of standard drills | Use standard drill sizes (fractional, metric, or letter) |
| 6 | Non-standard thread sizes | Requires thread milling instead of stock taps | Use standard metric coarse or UNC thread sizes |
| 7 | Flat bottom blind holes without specifying | Requires separate end mill operation after drilling | Accept drill-point bottom or explicitly callout flat bottom |
| 8 | Text or logos on machined surfaces | Requires engraving or EDM; adds significant time | Minimize text; use simple sans-serif fonts; specify depth |
| 9 | Undercuts that can be redesigned away | Requires special tooling and often manual operations | Redesign to eliminate undercuts wherever possible |
| 10 | No datum surfaces or fixturing features | Part cannot be securely held or accurately located | Add parallel faces, locating holes, or reference datums |
| 11 | Specifying surface finish on all surfaces | Entire part must be finish-machined | Only specify finish where functionally required |
| 12 | Overly complex geometry that could be split into two parts | Extreme tool access issues and many setups | Consider multi-piece assembly for complex geometries |
| 13 | Holes drilling into angled or curved surfaces | Drill walks on angled entry; poor position accuracy | Add a local flat (spot face) or specify centering operation |
| 14 | Thread depth exceeding 2x diameter in blind holes | High risk of tap breakage and difficult chip evacuation | Limit thread depth to 1.5-2.0x diameter |
| 15 | Inconsistent fillet radii throughout the part | Excessive tool changes increase cycle time | Standardize on 2-3 fillet radii matching standard tool sizes |

# 13. Cost Optimization Strategies

This section provides actionable strategies for reducing CNC machining cost without compromising functional requirements.

## 13.1 Design-Level Strategies

- Minimize number of setups: Design features accessible from as few orientations as possible. Each setup adds 15-30 minutes of non-cutting time.

- Use standard stock sizes: Design parts to be cut from standard bar, plate, or tube sizes. Minimize face-off material and reduce raw material waste.

- Avoid deep pockets: Reducing pocket depth from 50 mm to 25 mm can cut cycle time by 60% or more due to fewer passes and less material removal.

- Design near-net-shape: Start with stock that closely matches the final part envelope. Consider extrusions, castings, or forgings as raw stock for high-volume parts.

- Reduce the number of unique tools needed: Each tool change adds 5-15 seconds per part. Standardize features to use the same tools.

- Relax tolerances: Moving from +/- 0.025 mm to +/- 0.13 mm on non-critical features can reduce machining time by 30-50%.

- Batch-friendly design: Design parts to be machined in multiples per setup (gang machining) or from a single workpiece (multi-part nesting).

- Avoid cosmetic machining: Leave non-visible surfaces as-machined. Decorative features (chamfered edges, polished surfaces) add significant time.

## 13.2 Material-Level Strategies

- Choose free-machining alloys: Switching from 304 stainless steel to 303 stainless steel can reduce cycle time by 30-40% with negligible mechanical property difference.

- Pre-hardened materials: Using pre-hardened 4140 (28-32 HRC) eliminates post-machining heat treatment for many applications.

- Material substitution analysis: Aluminum can often replace steel for non-structural components, cutting machining time by 60-70% and reducing weight.

- Consider material form: Tube stock for hollow cylindrical parts, angle stock for bracket-shaped parts, and extruded profiles for channel shapes reduce material removal significantly.

# 14. Drawing and Documentation Best Practices

Clear, complete engineering drawings are essential for successful manufacturing. Ambiguous or incomplete drawings lead to delays, errors, and increased cost.

## 14.1 Required Drawing Information

- Material specification: Full material callout including alloy, temper/condition, and specification (e.g., Aluminum 6061-T6 per AMS QQ-A-250/11).

- General tolerance block: Define default tolerances for all features not explicitly dimensioned.

- Surface finish: Default surface finish (typically Ra 3.2 um as-machined) plus specific callouts on critical surfaces.

- Heat treatment requirements: Specify the desired condition and hardness range (e.g., Heat treat to 28-32 HRC per AMS 2759).

- Surface treatment / coating: Full callout including spec, type, class, and color where applicable (e.g., Anodize Type II, Class 2, Black per MIL-A-8625).

- Thread callout: Thread size, class, and depth for all threaded features.

- Deburr and break edges: Standard note: Break all sharp edges 0.25 mm max unless otherwise specified.

- Part marking: Specify method (engrave, etch, stamp), location, size, and content for part identification.

- Inspection requirements: Identify critical dimensions requiring first article inspection (FAI) or certificate of conformance (CoC).

# 15. Reference Tables

## 15.1 Standard Metric Drill Sizes

The following metric drill sizes are universally stocked by CNC machine shops. Using these sizes avoids reaming and reduces cost.

| Diameter (mm) | Diameter (mm) | Diameter (mm) | Diameter (mm) |
| --- | --- | --- | --- |
| 1.0 | 3.5 | 7.0 | 13.0 |
| 1.5 | 4.0 | 8.0 | 14.0 |
| 2.0 | 4.5 | 9.0 | 16.0 |
| 2.5 | 5.0 | 10.0 | 18.0 |
| 3.0 | 6.0 | 12.0 | 20.0 |

## 15.2 Tap Drill Sizes (Metric Coarse Threads, ~75% Thread)

| Thread | Tap Drill (mm) | Thread | Tap Drill (mm) |
| --- | --- | --- | --- |
| M3 x 0.5 | 2.5 | M10 x 1.5 | 8.5 |
| M4 x 0.7 | 3.3 | M12 x 1.75 | 10.2 |
| M5 x 0.8 | 4.2 | M14 x 2.0 | 12.0 |
| M6 x 1.0 | 5.0 | M16 x 2.0 | 14.0 |
| M8 x 1.25 | 6.8 | M20 x 2.5 | 17.5 |

## 15.3 Standard End Mill Diameters

Use these standard diameters when specifying internal corner radii and pocket widths to ensure standard tooling can be used.

| Metric (mm) | Imperial (inches) | Metric (mm) | Imperial (inches) |
| --- | --- | --- | --- |
| 1.0 | 1/16" (1.59 mm) | 10.0 | 3/8" (9.53 mm) |
| 2.0 | 3/32" (2.38 mm) | 12.0 | 1/2" (12.7 mm) |
| 3.0 | 1/8" (3.18 mm) | 16.0 | 5/8" (15.88 mm) |
| 4.0 | 3/16" (4.76 mm) | 20.0 | 3/4" (19.05 mm) |
| 5.0 | 1/4" (6.35 mm) | 25.0 | 1" (25.4 mm) |
| 6.0 | 5/16" (7.94 mm) | 32.0 | 1-1/4" (31.75 mm) |
| 8.0 | N/A | 40.0 | 1-1/2" (38.1 mm) |

## 15.4 Unit Conversion Quick Reference

| Conversion | Value | Conversion | Value |
| --- | --- | --- | --- |
| 1 inch | 25.4 mm | 1 mm | 0.03937 inches |
| 1 thou (0.001") | 0.0254 mm | 0.01 mm | 0.000394 inches |
| Ra 1 um | ~40 uin | Ra 3.2 um | ~125 uin |
| 1 N·m | 8.85 in·lbf | 1 ft·lbf | 1.356 N·m |
| 1 MPa | 145 psi | 1 ksi | 6.895 MPa |
| 1 kg | 2.205 lbs | 1 lb | 0.4536 kg |

# Appendix A: DFM Review Workflow

The following workflow outlines the recommended DFM review process for new part designs.

- Initial Design Concept: Complete 3D model with all functional features defined. Material selected. Operating loads and environment documented.

- Internal DFM Self-Check: Designer reviews part against guidelines in Sections 4 through 9 of this document. All feature dimensions checked against recommended values. Tolerances reviewed using Section 5 guidelines.

- Manufacturing Consultation: 3D model and preliminary drawing sent to manufacturing engineering or preferred CNC vendor for DFM review. Specific questions about challenging features highlighted.

- DFM Feedback Integration: Design modifications made based on manufacturing feedback. Trade-offs documented (e.g., feature cost vs. functional requirement).

- Final Drawing Release: Complete drawing with all information per Section 14. GD&T applied per Section 5. Material, finish, and treatment specifications complete.

- First Article Inspection (FAI): First production part inspected against drawing. Any out-of-tolerance features reviewed for drawing error vs. manufacturing issue. Drawing or process updated as needed.

# Appendix B: Glossary of CNC Machining Terms

| Term | Definition |
| --- | --- |
| Chatter | Vibration between the tool and workpiece that produces a rough surface finish and tool wear; often caused by thin walls, long tool extensions, or aggressive cutting parameters. |
| Chip Load | The thickness of material each cutting edge removes per revolution. Optimal chip load balances surface finish, tool life, and material removal rate. |
| Climb Milling | Milling where the cutter rotation is in the same direction as feed travel. Preferred for CNC machines; produces better finish and longer tool life. |
| Conventional Milling | Milling where the cutter rotation opposes feed direction. Causes more deflection and heat but is sometimes preferred for hard materials. |
| Depth of Cut (DOC) | The axial depth of material engagement per pass in milling, or radial depth in turning. |
| Dwelling | Holding the tool at a position momentarily. Used to improve surface finish at the bottom of bored holes. |
| End Mill | A rotary cutting tool for milling operations. Available in flat-end, ball-nose, corner-radius, and specialty profiles. |
| Feed Rate | The speed at which the cutting tool advances through the workpiece, typically in mm/min. |
| Fixture | A workholding device that locates, supports, and clamps the workpiece during machining. |
| G-Code | The programming language used to control CNC machines. Defines tool paths, speeds, feeds, and operations. |
| Guide Bushing | A precision bushing on Swiss-type lathes that supports the workpiece adjacent to the cutting zone. |
| Helix Angle | The angle of the cutting flutes on an end mill. Higher helix angles (40-45 deg) produce smoother cuts in aluminum. |
| Interpolation | Synchronized multi-axis movement to create circular or contour tool paths. |
| Live Tooling | Rotating cutting tools on a lathe that enable milling, drilling, and tapping without removing the part from the chuck. |
| Peck Drilling | A drilling cycle where the drill retracts periodically to clear chips. Essential for deep holes. |
| Runout | The total variation in the position of a rotating surface measured at a fixed point. Affects concentricity and surface finish. |
| SFM / SMM | Surface Feet per Minute / Surface Meters per Minute. The speed at which the cutting edge moves across the workpiece surface. |
| Spot Drill / Center Drill | A short rigid drill used to create a starting point (pilot hole) for a subsequent drill to prevent walking. |
| Step-Over | The radial distance between adjacent tool paths in area milling. Smaller step-over gives better surface finish but longer cycle time. |
| Tailstock | A support at the opposite end of a lathe from the chuck. Provides center support for long workpieces. |
| TIR (Total Indicated Runout) | The full range of indicator reading when measuring a rotating feature. Used to express concentricity and coaxiality. |
| Tool Deflection | The bending of a cutting tool under cutting forces. Increases with tool length and decreases with tool diameter. |
| Work Envelope | The maximum XYZ travel of a CNC machine. Parts must fit within this volume. |